|  | Simulation of FEA_finger_joint_v3.2 Date: Thursday, April 1, 2021 Designer: Solidworks Study name: Static 1 Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 4 Connector Definitions 5 Contact Information 5 Mesh information 6 Sensor Details 7 Resultant Forces 7 Beams 7 Study Results 8 Conclusion 11. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: FEA_finger_joint_v3.2 Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Split Line2; Solid Body; Mass:0.0085645 kg Volume:4.12549e-06 m^3 Density:2,075.99 kg/m^3 Weight:0.0839321 N; C:\Users\luisa\Documents\GrabCAD\Lunar Dust Challenge\Current\Mechanical\v3_tiz\FEA\FEA_finger_joint_v3.2.SLDPRT Apr 1 22:46:40 2021.

# Study Properties Study name; Static 1. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; FFEPlus. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\luisa\Documents\GrabCAD\Lunar Dust Challenge\Current\Mechanical\v3_tiz\FEA).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; 99.5% Aluminum Oxide. Model type:; Linear Elastic Isotropic. Default failure criterion:; Mohr-Coulomb Stress. Yield strength:; 2.17185e+07 N/m^2. Tensile strength:; 2.17185e+07 N/m^2. Compressive strength:; 2.99922e+07 N/m^2. Elastic modulus:; 3e+11 N/m^2. Poisson's ratio:; 0.22. Mass density:; 2,075.99 kg/m^3. Thermal expansion coefficient:; 8e-06 /Kelvin.; SolidBody 1(Split Line2)(FEA_finger_joint_v3.2). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -0.000433497; -119.998; -0.0014181; 119.998. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s). Type:; Apply normal force. Value:; -120 N..

# Connector Definitions No Data

# Contact Information No Data

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.802247 mm. Tolerance; 0.0401124 mm. Mesh Quality Plot; High. ## Mesh information - Details Total Nodes; 83132. Total Elements; 54412. Maximum Aspect Ratio; 13.498. % of elements with Aspect Ratio < 3; 99.8. % of elements with Aspect Ratio > 10; 0.00919. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:03. Computer name:.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -0.000433497; -119.998; -0.0014181; 119.998. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 8.040e+01 N/m^2 Node: 70215; 1.301e+07 N/m^2 Node: 81540. FEA_finger_joint_v3.2-Static 1-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00 mm Node: 1; 5.728e-04 mm Node: 3165. FEA_finger_joint_v3.2-Static 1-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 8.533e-11 Element: 7062; 3.040e-05 Element: 32892. FEA_finger_joint_v3.2-Static 1-Strain-Strain1. Name; Type. Displacement1{1}; Deformed shape. FEA_finger_joint_v3.2-Static 1-Displacement-Displacement1{1}. Name; Type; Min; Max. Factor of Safety1; Mohr-Coulomb Stress; 1.259e+00 Node: 81540; 2.330e+05 Node: 70206. FEA_finger_joint_v3.2-Static 1-Factor of Safety-Factor of Safety1. Name; Type. Design Insight1; Design Insight. FEA_finger_joint_v3.2-Static 1-Design Insight-Design Insight1.

# Conclusion